TESTING PROCEDURE

### STEP 1

### Before Opening the Debug Application:

- Download the application from the following link:

- https://drive.google.com/drive/folders/1sJkkr4wi0dE8rRRF6e2P8BvkCDZE7fXo?usp=sharing

- Once the download is complete, go to your Downloads folder.

- Double-click the SERIAL PORT application to open it.

## STEP 2

### In the Debug Application:

- Click the Serial Open button.

- Set the:

  - Baud Rate (as 115200)

  - COM Port (matching the connected device)

- Click the Connect button to establish the connection.

## STEP 3

Once connected, the IDN command will automatically be sent, and you will receive an acknowledgment response from the device.

## STEP 4

- Click the LCD POWER button.

- - Initially, this sets the pin HIGH.

- - Next click the LCD TEST button, LCD will On. Bars will appear sequentially in both horizontal and vertical directions.

## STEP 5:

- Use the LCD BACKLIGHT control to increase or decrease the LCD brightness according to your preference

## To turn OFF the LCD, click the LCD POWER and LCD TEST buttons again:

- These buttons act as toggles — clicking them again will turn OFF the LCD POWER and LCD TEST functions

## STEP 6:

## BLUETOOTH:

## Click the BLE button.
First click: Powers ON the BLE module and starts advertising.

## Click again: Stops advertising (toggle behaviour).

- STEP 7:

- SWOUT

## Click the SWOUT button.

- - Initially, this sets the pin HIGH.

- - Clicking the same button again will pull the pin LOW.

## STEP 8:

- KEYPAD

Click the KEYPAD READ button to enable keypad detection.

When you press a key, the pressed key is displayed.

Click the KEYPAD READ button again to stop keypad detection.

## STEP 9:

- GPS

- Click the GPS EN button

- This also pulls the GPS power pin HIGH.

- Click the GPS RESET button

- This pulls the GPS power pin HIGH.

- Click the GPS READ button

- This starts continuous reading of GPS NMEA data.

## STEP 10:

Click the GPS READ button again to stop reading GPS NMEA data.

Then click the GPS EN and GPS RESET buttons to pull the GPS power pin LOW.

## STEP 11:

### Memory Power:

- Click the MEMORY button

  - The memory power pin will be pulled HIGH.

  - An ACK message will be shown for confirmation.

- Click the MEMORY button again

  - The memory power pin will be pulled LOW.

### STEP 12:

### FLASH Operation:

### Click the FLASH READ button:

  - Displays the value and address.

  - Writing to flash is performed in the same way (using a similar process).

NOTE:

Analog Enable button should be HIGH before testing Relay and Both Internal & External ADC.

## STEP 13:

### Relay Control:

- Click any RELAY button (Relay 1 to Relay 9).

  - The selected relay will turn ON.

  - An ACK message will be displayed to confirm the action.

- Repeat this process to test other relays.

STEP14:

ADC TYPE

First, click the 'ADC TYPE' button to select the Internal ADC. Clicking the same button again will switch to the External ADC.

STEP 15:

ADC Sample Length & Clock Rate

ADC Sample Length

- Enter the desired sample length.

- Both the internal and external ADCs will use this length.

ADC Clock Rate

- Enter the desired clock rate (in kSPS).

- This sets the ADC sampling clock frequency.

STEP16:

Internal ADC – DC Mode

- For DC mode, the necessary relay channels must be turned ON.

- For Channels 0 & 1, enable the following relays:
KCH1, KCRNG1, and KCAL.

- For Channels 2 & 3, enable the following relays:
KCH2, KCRNG2, and KCAL

- For Channel 4, enable the following relay:
KCH3.

- For Channel 5, enable the following relays:
KCH4, KCRNG4, and KCH4S.

STEP17:

Internal ADC – AC Mode

- For AC mode, the necessary relay channels must be turned ON.

- For Channels 0 & 1, enable the following relays:
KCH1, KCRNG1, and KCAL

- For Channels 2 & 3, enable the following relays:
KCH2, KCRNG2, and KCAL

- For Channel 4, enable the following relay:
KCH3.

- For Channel 5, enable the following relays:
KCH4, KCRNG4, and KCH4S.

STEP 18:

External ADC – DC Mode

- For DC mode, the necessary relay channels must be turned ON.

- For Channels 0 & 1, enable the following relays:
KCH1, KCRNG1, and KCAL

- For Channels 2 & 3, enable the following relays:
KCH2, KCRNG2, and KCAL

- For Channel 4, enable the following relay:
KCH3.

- For Channel 5, enable the following relays:
KCH4, KCRNG4, and KCH4S.

STEP 19:

External ADC – AC Mode

- For DC mode, the necessary relay channels must be turned ON.

- For Channels 0 & 1, enable the following relays:
KCH1, KCRNG1, and KCAL

- For Channels 2 & 3, enable the following relays:
KCH2, KCRNG2, and KCAL

- For Channel 4, enable the following relay:
KCH3.

- For Channel 5, enable the following relays:
KCH4, KCRNG4, and KCH4S.

STEP 20:

Power Mode

Power Mode 2:
Sets the controller to low-frequency mode via Timer.
This reduces the operating frequency using a timer to lower power consumption.

Power Mode 4:
Sets the controller to deep sleep mode via BURTC (Backup Real-Time Counter).
After the delay, it will automatically restart.
This mode puts the controller in a very low power state and restarts it after a set period.